TECHNICAL REFERENCE FOR ORGANISER FLASH DATAPAKS

Date of issue: 14/1/91             Document Version: 1.0

This is a supplement to the Organiser II Technical Manual.

INTRODUCTION

Flash Datapaks on the Organiser represent a substantial

improvement in storage technology.  There are four major

advantages:

- significantly less power used when writing,

- non-volatile,

- can be formatted in place,

- bigger than available EPROMs.

This document explains the technical details that enable

this new technology to be implemented.

HARDWARE

Flash Datapaks consist of either one or two Flash chips on

a board with associated regulator, counters and logic.  All

circuitry is CMOS so reading requires very little power.

READING

Flash Datapaks are paged and segmented.  Segments control

address lines A14 to A18 (16K byte blocks) and pages

control lines A8 to A13 (256 byte blocks).  So to get to

the last byte requires the segment to be set, 63 toggles of

the page and 255 toggles of the address.  This will take in

the order of 1 millisecond.

All Organiser models can read data from Flash Datapaks

except CMs which are not able to recognise segmented packs.

WRITING

Writing is done in a completely different way to EPROMs and

a special software driver must be present.  If the Flash

driver is not present and a write is attempted a "READ ONLY

PACK" error is reported.

It is impossible to quantify the power saving compared to

EPROMs.  It depends primarily on the length of the data

being written.  The longer the data the greater the saving.

As a generalisation a user can expect an order of magnitude

improvement though 2 orders is quite possible.

FORMATTING

Flash Datapaks can be formatted in place.  To do this each

byte must be written to zero and then for the one or two

chips present they must commanded to erase back to FF.

When doing this each byte must be checked to ensure that is

has been erased properly.  If not it tries another erase

until either all bytes are FF or the chip is judged to have

failed.

In Flash driver Version 1.3 and greater packs can be

formatted by calling the OPL procedure FLSHFORM:("B",0)

where the first parameter is the slot (B or C) and the

second is whether a display is required.  If the second is

non-zero then the standard display is given otherwise the

screen is not altered.

Formatting does consume a noticeable amount of power and it

is suggested that a power supply unit is connected.

WRITE PROTECT

All Flash Datapaks have their write protect bit set to zero

so that unless the Flash driver is present no writing can

take place.  In order that Flash Datapaks can be write

protected the most significant bit of byte 8 of the header

is used as the alternate write protect bit.  When a pack is

freshly formatted it is always 1 (ie write enabled).

The format of the header is basically the standard bootable

header:

--------------------------------------------------------------------

| TYPE | SIZE | CODE | DEV | VER | PRIOR | BOOT ADDR | RANDOM WORD |

--------------------------------------------------------------------

TYPE           bit 0     Not used, always 0

bit 1     EPROM (ie not RAMPAK) always set to 1

bit 2     PGCPK always set to 1 as paged

bit 3     RDWRT always set to 0

bit 4     BOOT generally set to 0 as pack bootable

bit 5     COPYPK generally set to 1 as pack is copyable

bit 6     NYIMPL set 0 to indicate flash

bit 7     MK1PAK set to 0 as not MKI pack

The default type bye is $26.

SIZE           $20 for 256K pack.

CODE           $01

DEV & PRIOR    $F8

BOOT           $19 default

RANDOM WORD    The most significant bit is used as the real

write protect bit

THE FLASH DRIVER

The driver software comes as a bootable device on every

standard Flash Datapak and on the Flash Formatter packs.

It is different from other drivers in that it is permanent,

it does not get removed when On/Clear is pressed at the top

level even if the bootable pack is no longer present.

In order to achieve this it assumes that it is the first

item booted in.  It has priority $F8.  If something else is

already booted in and not also a permanent device it will

be removed from the device list.  This should, of course,

never happen.

To make a device permanent two alternative strategies

should be followed depending on whether the code is booted

in the low 7K or in the PERMCELL.

If it is booted in the low 7K then make 'dva_bot' the same

as 'dva_top'.  Remember both the old values.  When

attempting to remove check that both 'dva_top' and

'dva_bot' are equal and the same as the old 'dva_top'.

Refuse if they are not.  If they are then replace both with

the old 'dva_bot'.

If it is booted in the PERMCELL then remember the old base

value (at $2000) and make it equal to the value at $2002.

When attempting to remove check that the value at $2000 &

$2002 is as you left it then make $2000 the old value.

Then push the address of 'al$zero' on the stack and exit

(get the address by looking in the table pointed to by

'bta_vect').

To ensure that you are not booted in more than once use a

bit in 'dvt_spar'.  To be allocated a bit talk to Support.

Flash sets bit 2 of $2178 when it is present.

It re-vectors the following OS routines:

pk$save       fl$writ        fl$eras        fl$copy

fl$deln       fl$bdel        fl$renm        dv$vect

dv$lkup       dv$clsr        pk$setp

To remove the driver all other drivers must be removed (by

pulling out all bootable packs and then pressing On/Clear

at the top level) and REMOVE used in the FLASH menu.  All

other drivers must be removed to avoid re-revectoring

problems.  If a device is still present the REMOVE fails

with the message "BAD DEVICE CALL".  If other permanent

drivers are released then they should be removed in this

way, one at a time, and in the reverse order to which they

were inserted.

This is a complete new procedure and care must be taken to

explain it to users when an upgrade of the driver is

released.  There is no exterior indication that the driver

has been upgraded.

Every time the Organiser is rebooted he name FLASH is re-

inserted in the menu.  This is to avoid the situation where

FLASH has been deleted from the menu.

THE MULTIBOOT FLASH DRIVER

The multi-boot Flash driver allows all long records

following to be booted in.  This means that other OPL

extensions can be added without altering the Flash Driver.

It is an extension to the standard Flash Driver.

It requires about 500 bytes of extra code some of which

will be taken out of the PERMANENT cell.  For this reason

it is not included in the standard driver.

The first time it boots as well as making itself permanent,

it boots all the long records (02 80 type that are assumed

to be .BINs) immediately after the bootable code on the

pack.  The second time (ie when the driver is already

present) it will be boot into the PERMCELL (the existing

flash driver will already take up most of the lower 7K).

Once it has determined that the flash driver is already

present it minimises its size and boots the following long

records.

To generate a pack to utilise this feature either use

custom software or release 3.20 or above of BLDPACK.

WARNING:  BLDPACK is unable to adjust any absolute pack

addressing in the .BINs that it strings together.  The main

use of absolute pack addresses is for loading overlays.

There are a variety of ways round this problem if it is

absolutely necessary.

OTHER FACILITIES

MAKE - version 3.20 and above will allow Flash Datapaks to

be made in the ordinary way.

FMAKE - uses the copier parallel interface to the IBM and

allows 256k Flash Datapaks to be made from a 256K image.

Copier File Transfer Utilities - serial and parallel utilities

in beta test, on this disk in COPIER directory.

VERSIONS

V1.2      RELEASED 1st November 1990

First release.

V1.3b      Not RELEASED ? January 1991

OPL call FLSHFORM supported.

Multi Boot version released.

V1.5 (FDRIVE15.BIN) & M1.5 (FDRIVM15.BIN) Released Dec 91

New versions of normal flash driver and multi-boot flash driver

to support new flash chips. Also downwardly compatible. Please

read following for details of upgrading versions. Note, M1.5 is

the multi-boot version.

Flash Datapak software versions

-------------------------------

If you have more than one Flash Datapak, they may have different

versions of the controlling software. You MUST install whichever

version is the latest. The original version of the software did not

display a version number; subsequent versions display a version

number when you select the `Flash' option. (On XPs, the number is on

the end of the menu.)

To use a second Flash Datapak, first note the version of your current

software, then remove the software (as explained in the Flash Datapak

leaflet). Now install the software from the new Datapak, and note its

version number. It will probably be the same version, or a later one.

You can now use both Flash Datapaks.

The new Flash Datapak may not work if you do not do this.

In the unlikely event that the new software is an earlier version

that the software from your original Datapak, remove the new software

and re-install your previous software.

If you do have different versions of the software, then having

installed the most recent version, you should format any Flash

Datapaks which have the old version. (Otherwise, when you come to

format the newer Datapak, the formatting process would copy the older

version of the software from the older Datapak onto the newer one,

and the newer Datapak might not work.) You can copy any data from the

older Datapak - perhaps to the newer one - before formatting it.

Flash Datapak Formatters contain the same formatting software as

Flash Datapaks themselves, and you can check the version number of

the software in the same way. You must not use a Flash Datapak

Formatter to format a Flash Datapak if the Datapak itself has a later

version of the software. In this event, return your Flash Datapak

Formatter to Psion for an upgrade.